# BC CSR Screening Report

Date generated: 10/12/2026

Site: Demonstration only; assessment mode: both

Sample count: 2. Samples are compared individually; no averaging.

Groundwater used for drinking water: Assumed applicable for this fictional demonstration.

Educational use only. Not professional environmental advice or a formal ESA. Verify current BC CSR requirements and pathway applicability before relying on results.

Dataset: BC Contaminated Sites Regulation — Schedule 3.1; source snapshot: Source tables verified 2026-10-11; consolidation 2026-10-06

Official regulation: https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/375_96_00

Schedule 3.1 soil only. Unassessed factors and unsupported conditions prevent a confirmed pass. Source URLs: https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part1 ; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/159_2025_Sch%203.1-part2 ; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part3

# Land Use Summary

Current land use: Residential Low Density

Proposed land use: Industrial

Scenarios evaluated: Current: Residential Low Density / Proposed: Industrial

Selected thresholds: Schedule 3.1 soil standards (Parts 1–3)

# Results Summary

1 exceedance(s), 4 evaluated scenario comparison(s), 2 scenario comparison(s) requiring review, 1 selected contaminant(s).

| Sample / contaminant | CAS | Land use / factor | Result | Unit | Standard | Unit | Status | Ratio | Source / review note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Former fuel storage / BH-01 / S01 / 0.5–1 m / 2026-10-12 / Anthracene | 120-12-7 | Current RLLD — Intake of contaminated soil | 3 | mg/kg | 10 000 | µg/g | Pass | 0.0003× | Schedule 3.1 Part 1 Matrix 1; Intake of contaminated soil; column 6; PDF page(s) 1; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part1; Overall soil scenario: Exceeds |
| Former fuel storage / BH-01 / S01 / 0.5–1 m / 2026-10-12 / Anthracene | 120-12-7 | Current RLLD — Groundwater used for drinking water | 3 | mg/kg | NS | µg/g | No numerical standard | — | Schedule 3.1 Part 1 Matrix 1; Groundwater used for drinking water; column 6; PDF page(s) 1; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part1; Overall soil scenario: Exceeds |
| Former fuel storage / BH-01 / S01 / 0.5–1 m / 2026-10-12 / Anthracene | 120-12-7 | Current RLLD — Toxicity to soil invertebrates and plants (controlling standard) | 3 | mg/kg | 2.5 | µg/g | Exceeds | 1.2× | Schedule 3.1 Part 1 Matrix 1; Toxicity to soil invertebrates and plants; column 6; PDF page(s) 1; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part1; Overall soil scenario: Exceeds |
| Former fuel storage / BH-01 / S01 / 0.5–1 m / 2026-10-12 / Anthracene | 120-12-7 | Proposed IL — Intake of contaminated soil | 3 | mg/kg | > 1 000 mg/g | mg/g | Inconclusive | — | Schedule 3.1 Part 1 Matrix 1; Intake of contaminated soil; column 9; PDF page(s) 1; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part1; Source conditions or non-numerical notation require professional review; automatic treatment is not supported. Overall soil scenario: Inconclusive |
| Former fuel storage / BH-01 / S01 / 0.5–1 m / 2026-10-12 / Anthracene | 120-12-7 | Proposed IL — Groundwater used for drinking water | 3 | mg/kg | NS | µg/g | No numerical standard | — | Schedule 3.1 Part 1 Matrix 1; Groundwater used for drinking water; column 9; PDF page(s) 1; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part1; Overall soil scenario: Inconclusive |
| Former fuel storage / BH-01 / S01 / 0.5–1 m / 2026-10-12 / Anthracene | 120-12-7 | Proposed IL — Toxicity to soil invertebrates and plants (controlling standard) | 3 | mg/kg | 30 | µg/g | Pass | 0.1× | Schedule 3.1 Part 1 Matrix 1; Toxicity to soil invertebrates and plants; column 9; PDF page(s) 1; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part1; Overall soil scenario: Inconclusive |
| Former fuel storage / BH-01 / S02 / 1.5–2 m / Anthracene | 120-12-7 | Current RLLD — Intake of contaminated soil | < 1 | mg/kg | 10 000 | µg/g | Pass | 0.0001× | Schedule 3.1 Part 1 Matrix 1; Intake of contaminated soil; column 6; PDF page(s) 1; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part1; Overall soil scenario: Pass |
| Former fuel storage / BH-01 / S02 / 1.5–2 m / Anthracene | 120-12-7 | Current RLLD — Groundwater used for drinking water | < 1 | mg/kg | NS | µg/g | No numerical standard | — | Schedule 3.1 Part 1 Matrix 1; Groundwater used for drinking water; column 6; PDF page(s) 1; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part1; Overall soil scenario: Pass |
| Former fuel storage / BH-01 / S02 / 1.5–2 m / Anthracene | 120-12-7 | Current RLLD — Toxicity to soil invertebrates and plants (controlling standard) | < 1 | mg/kg | 2.5 | µg/g | Pass | 0.4× | Schedule 3.1 Part 1 Matrix 1; Toxicity to soil invertebrates and plants; column 6; PDF page(s) 1; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part1; Overall soil scenario: Pass |
| Former fuel storage / BH-01 / S02 / 1.5–2 m / Anthracene | 120-12-7 | Proposed IL — Intake of contaminated soil | < 1 | mg/kg | > 1 000 mg/g | mg/g | Inconclusive | — | Schedule 3.1 Part 1 Matrix 1; Intake of contaminated soil; column 9; PDF page(s) 1; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part1; Source conditions or non-numerical notation require professional review; automatic treatment is not supported. Overall soil scenario: Inconclusive |
| Former fuel storage / BH-01 / S02 / 1.5–2 m / Anthracene | 120-12-7 | Proposed IL — Groundwater used for drinking water | < 1 | mg/kg | NS | µg/g | No numerical standard | — | Schedule 3.1 Part 1 Matrix 1; Groundwater used for drinking water; column 9; PDF page(s) 1; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part1; Overall soil scenario: Inconclusive |
| Former fuel storage / BH-01 / S02 / 1.5–2 m / Anthracene | 120-12-7 | Proposed IL — Toxicity to soil invertebrates and plants (controlling standard) | < 1 | mg/kg | 30 | µg/g | Pass | 0.0333× | Schedule 3.1 Part 1 Matrix 1; Toxicity to soil invertebrates and plants; column 9; PDF page(s) 1; https://www.bclaws.gov.bc.ca/civix/document/id/complete/statreg/133_2022_Sch%203.1-part1; Overall soil scenario: Inconclusive |

# Recommendation

One or more evaluated scenario comparisons exceed the selected Schedule 3.1 thresholds. A Detailed Site Investigation may be needed to characterize the location, extent, and degree of contamination. 2 soil scenario(s) have unresolved applicability or source conditions. No confirmed pass is made for those scenarios.